HMI INTERFACE DESCRIPTION

ORMIN / ORMECO Monitoring System

INITIAL CUSTOMER SUBMISSION

1. Purpose

This document provides a brief description of the Human–Machine Interface (HMI) supplied for the ORMIN / ORMECO monitoring system. The HMI provides live equipment monitoring, historical trends, fuel-consumption analysis and system-status information.

2. System Interface

The HMI is a web-based application connected to the monitoring system through the following interfaces:

Field communication: Modbus TCP

Live-data processing: HMI backend server

Historical-data storage: SQLite database

Operator interface: Standard web browser

Data update: Automatic periodic refresh

| Sensors / PLC | Modbus TCP | HMI Backend | Web HMI Display |
| --- | --- | --- | --- |

3. HMI Screens

| Screen | Description |
| --- | --- |
| Overview | Summary of the main operating parameters for D/G No. 1 to D/G No. 4. |
| Engine | Detailed operating parameters for the selected engine. |
| Engine Status | Engine operating and status indications. |
| Device Status | Operating condition of monitored equipment and devices. |
| P&ID | Process values and equipment status on the system diagram. |
| Pressure Trend | Historical pressure values compared with engine load. |
| Exhaust Temperature Trend | Cylinder and turbocharger temperatures compared with engine load. |
| Fuel Consumption | D.O. and H.O. inlet flow, return flow and calculated consumption. |
| Administration | Authorized signal, collector and database configuration functions. |

4. Displayed Information

The HMI displays operating information including:

Engine power, speed and running hours

Fuel-oil, lubricating-oil, cooling-water and intake-air parameters

Cylinder exhaust and turbocharger temperatures

Fuel inlet, return and transfer flow

Equipment running, stopped, trip and fault conditions

Communication status and historical trends

5. Status and Communication

The HMI uses values, text, symbols and colours to identify normal, stopped, warning, trip, fault and communication-unavailable conditions. Live information is acquired through Modbus TCP. Historical information is stored locally for trend and fuel-consumption display.

If communication is interrupted, the HMI provides a communication-status indication. Displayed information must not be considered current until communication is restored.

6. Scope and Confirmation

The current HMI is intended primarily for monitoring, visualization and operational analysis. Final signal addresses, equipment descriptions, engineering units, status colours, alarm limits and screen arrangements are subject to customer review and project commissioning.

Appendix A – HMI Screen Captures

Replace each grey placeholder with the applicable approved HMI screenshot before final submission.

A.1 Overview

A.2 Engine

A.3 Engine Status

A.4 Device Status

A.5 P&ID

A.6 Pressure Trend

A.7 Exhaust Temperature Trend

A.8 Fuel Consumption

A.9 Administration

.